TWEEQU (version 1.0)

Thermobarometry With Estimation of EQUilibration state

February 9, 1992

CONTENTS

1. RECOMMENDED CITATIONS

2. INTRODUCTION

Test file Description

3. HIGHLIGHTS FOR 'OLD-TIME' USERS

4. OVERVIEW OF SOFTWARE

5. SETUP/CONFIGURATION

6. CREATING MINERAL COMPOSITION FILES

7. DEFINITION OF SOLUTION PHASES

8. TWQ OPERATION

9. AUXILIARY PROGRAMS - OPERATION

PLOT.EXE

PRINTER.EXE

LASER.EXE

CLEAN.EXE

INTERSX.EXE

10. BATCH MODE OPERATION OF TWQ AND INTERSX

11. REFERENCES

APPENDIX I: PHASES IN THERMODYNAMIC DATABASE (JAN92.RGB)

APPENDIX II: TWQ PRINCIPLES AND EQUATIONS OF STATE

APPENDIX III: DESCRIPTION OF I/O FILES

APPENDIX IV: GRAPHICS CARD OPTIONS FOR USE WITH PLOT.EXE

-----------------------------------------------------------------

RECOMMENDED CITATIONS:

When publishing thermobarometric results calculated with this software,

please cite the program itself (Berman, 1991) as well as the thermodynamic

data used, including end-member properties (Berman, 1988; 1990) and the

solid solution models that you used (see below). Full references are listed

near the end of this document.

-----------------------------------------------------------------

INTRODUCTION

TWQ (version 1.0) replaces what used to be called PTAXSS. It is

basically the same program but has been renamed to better distinguish

it from the UBC distributed GE0CALC program called PTAX. The new

name, abbreviated from TWEEQU (Thermobarometry With Estimation of

EQUilibration state) emphasizes the important thermobarometric

application of being able to test the assumption that all mineral

compositions used in a calculation last equilibrated at the same P-T.

For the perfectly behaved system (all minerals equilibrate at same P,T;

perfectly known mineral compositions, standard state and mixing

properties), the calculated phase diagrams should have all equilibria

intersecting at a unique point. So much for perfection! The degree

of scatter in the intersections gives an indication of the validity of

these assumptions as well as a qualitative measure of the uncertainties in

the calculation. In many cases, this uncertainty can be substantially

reduced by omitting from the calculations minerals for which standard state

or mixing properties are poorly known. As we gain more confidence in

thermodynamic properties it should be possible to use this program to tell

whether all minerals equilibrated at the same P-T conditions. See Berman

(1991) for a detailed description of the philosophy of this program and

several examples of applications.

Several TEST files have been included on the distribution disc so that you may

make sure you are using the software correctly. Run TWQ and include the

elements: K Fe Mg Ca Al Si O H when requested. On the next screen include

the endmembers: ALMANDINE, ANNITE, ANORTHITE, GROSSULAR, MUSCOVITE, PHLOGOPITE,

PYROPE, ALPHA QUARTZ, SILLIMANITE, all with default activities equal to unity.

On the next screen, type 'a' to take the default solution models. Type

TEST.CMP for the file of compositional data. The program should calculate all

equilibria and produce a file PLOT.DAT that should be identical to the file

TEST.PLT on the distribution disc. View this file by typing PLOT PLOT.DAT.

Run CLEAN, take all defaults, and you should get a file named CLEAN.DAT that

is identical to the file TEST.CLN. View this file by typing PLOT CLEAN.DAT.

Run INTERSX, type TEST.CMP when asked for the file of compositional data, and

you should get averaged results in the file PLOT.AVG that are identical to

those in the file TEST.AVG. View these results using any text editor or by

typing TYPE PLOT.AVG at the DOS prompt. Full instructions for running all

programs are given below.

If you wish to be informed of upgrades of this software and associated data

(e.g. amphibole, biotite, orthopyroxene, clinopyroxene endmembers and

solution properties) send a BITNET/INTERNET message to: BERMAN@GSC.EMR.CA.

A Macintosh version of this program will be available sometime in 1996 from

Martin Engi. You can reach him at "engi@mps.unibe.ch".

Note to GE0-CALC users: Since 1992, PTAX has been completed.

This program combines GE0-CALC's earlier PTX and PTA programs into one

program, and adds the general solid solution equations incorporated in TWQ.

It will be distributed for a modest fee by:

GE0-CALC SOFTWARE

Department of Geological Sciences

University of British Columbia

Vancouver, British Columbia, Canada V6T 2B4

604-228-4653

The only difference between PTAX and the enclosed program, TWQ, is

that the latter program is designed specifically for thermobarometry

applications. It always calculates and plots all equilibria whether they

are stable or metastable (both are equally informative). You will have

to order PTAX through the above address if you wish to compute stable

phase diagrams with all metastable extensions removed.

-----------------------------------------------------------------

HIGHLIGHTS FOR `OLD-TIME' USERS (look below for details):

Changes in the latest TWQ version (v 1.01):

Bugs in fluid choice routine removed (discard results using fluids

with TWQ (v 1.0)

INTERSX correctly prints the number of independent reactions

The newest features that have been added to TWQ (v 1.0):

A new program (INTERSX) reads the plot file generated by TWQ and

computes an average P and T (or other axis variable) from all

intersections between curves, weighted towards those equilibria

with the highest deltaS, deltaV and smallest lnK', and which

intersect most orthogonally. To use this program after TWQ, type

INTERSX, and the output will be directed to the file PLOT.AVG as

well as to the video monitor.

A new plot program (PLOT) replaces PLOTS, and offers mouse support,

menu options, multiple file reads, change saves, and more graphics

card support. The PTAXCON.FIG file is set up for a VGA card and

monitor ("/e:5") at end of first line. Change this to "/e:4" for

EGA cards. See the more detailed notes below.

Multi-site mixing: Multisite nonideal mixing can now be handled in

a general way (see below) by defining mixing models in the file

SOLID.SLN (replaces MINERAL.SLN).

Note: Old CMP files will work with the new program, except for OPX

compositions. These need to be entered anew (see below).

Activities of any phase can be fixed in the CMP file that gives

mineral compositions (see below).

Operation in `batch' mode

New equation of state for CO2 that better extrapolates to high P

Mader & Berman (1991)

Endmember and mixing properties of amphiboles. See Mader & Berman (1992).

Other less recent changes that have been added over the last two years:

Bug which kept the hydration model for cordierite from being applied

has been corrected

Optional suppression of printing of the SUMMARY.DAT file

The early versions of PTXSS had unfortunate inconsistencies

in the properties of Fe-cordierite, staurolite, hercynite, and

chloritoid (FEB89.RGB). These were introduced when almandine

properties were revised to accommodate garnet mixing properties

(Berman, 1990). These inconsistencies have now been removed

(NOV89.RGB).

A typo on the Cp of hydrogen gas has now been corrected (FEB90.RGB)

The Nov89 version of PTAX did all mixed volatile calculations with

ideal H2O-CO2 mixing. This is now corrected.

Annite properties adjusted slightly (AUG90.RGB) due to analysis of

natural data yielding nonideal Ti,Al mixing parameters in biotite

(McMullin et al. 1991).

-----------------------------------------------------------------

OVERVIEW

TWEEQU is a software package consisting of a program  for calculation of

phase equilibrium curves, a thermodynamic database for minerals, and

auxiliary programs for plotting, printing, and averaging of computed

curves. The minimum hardware requirements for using this software are a

IBM-compatible personal computer with 640 K memory, math coprocessor, and

graphics card (CGA, EGA, VGA, or HERCULES).

The main part of this package is the FORTRAN 77 program TWQ which

calculates phase equilibria in pressure-temperature (P-T), temperature-

composition (T-XH2O-CO2), pressure-composition (P-XH2O-CO2), T-activity, P-

activity, or activity-activity sections. The program structure and

equations of state are discussed briefly in Appendix II.

The output of TWQ can be plotted for visual representation or inspected

from a table. The program evaluates all possible reactions in the user-

specified compositional and PTX space, and labels each equilibrium curve.

This software is distributed with the thermodynamic database of Berman

(1988), but options for a variety of other equations of state are provided

so that the programs can be used with different data sets. Run time options

include setting the P, T, X limits of the diagram, the selection of ideal

or non-ideal H2O-CO2 mixing, choice of solid solution models, the

specification of fixed phase activities,  and the choice of mineral

compositions read from an external file.

TWQ allows one to calculate any equilibrium curve using the variables P, T,

XCO2, or activity. The user should be ever aware that the accuracy of

calculated phase diagrams is a direct function of the quality of

thermodynamic input data. The database distributed with this software is

that presented by Berman (1988) and modified by Berman (1990) for the

system Na2O-K2O-CaO-MgO-FeO-Fe2O3-Al2O3-SiO2-TiO2-H2O-CO2. Details of the

procedure used to derive this data set are given by Berman et al. (1986).

Appendix I shows the list of minerals in the database along with their

chemical formulae, abbreviations, and reliability levels (see Berman, 1991

for a description of the reliability definitions). Users should be aware

that calculations involving reliability level 3 phases should be

interpreted with some caution, and that feedback on which phases seem to

give reasonable results and which don't is required (and appreciated!) in

order to improve some of our thermodynamic representations of these phases.

I strongly discourage users from replacing or modifying any parameters in

this data set because doing so will destroy the internal consistency of the

database. Users may wish to add phases which are not included in the data

set, and I advise reading the Berman et al. (1986) paper cited above for

guidelines of how best to derive reliable thermodynamic parameters.

Also note: This software has been tested extensively, and reasonable

efforts have been made to remove ambiguities and known bugs. Nevertheless,

I can safely assume that some problems have escaped our detection. If you

do have problems with this software, I ask your patience in dealing with

them and remind you that this software is distributed as a research tool,

not as a guaranteed commercial product. Please do not phone for help unless

absolutely necessary. I will be happy to assist you in solving problems,

but the most efficient way to receive help is to describe the problem in a

brief letter including a self-addressed envelope for return. Also include

a high density floppy disc that contains the input/output files (RESTART.RES,

SUMMARY.DAT, and PLOT.DAT) that were used/generated during the problem run. This

will provide me with the necessary information to solve your problem. The floppy

disc will be used to respond to you, and include any program/data modifications

that are necessary.

The most common steps in using this software package are:

1) Setup computer for TWQ operation

2) Use text editor to make files containing analytical data for minerals

3) Use TWQ.EXE to calculate phase equilibria

4) Use CLEAN.EXE to improve legibility of phase diagram

5) Use PLOT.EXE for an on-screen preview of phase diagram

6) Use PRINTER.EXE or LASER to make high resolution hard copy

7) Use INTERSX.EXE to compute average thermobarometric results

Each of these steps is described in detail below.

-----------------------------------------------------------------

SETUP/CONFIGURATION

Most users will want to keep this software in a directory which I assume

will be called `TWQ'. Use the DOS command `MKDIR TWQ' to make this

directory, and then `CD TWQ' to change to this directory. Copy all files on

the two distribution discs to your hard drive using the DOS command: `COPY

a:*.*'.

Before you run TWQ, you must configure your system to allow a minimum of 20

files to be open. Using any line editor, edit the DOS file `CONFIG.SYS'.

Add the line:

FILES=20

If you do not have the `CONFIG.SYS' file in your root directory, you must

create first it (see your DOS manual for details). Before running PT-

System, you need to restart your computer so that this file is processed.

Press the `CTRL', `ALT', and `DEL' keys simultaneously to reboot your

system, or turn your computer off and then on again.

The file `PTAXCON.FIG' contains information on lines 1-4 related to the

hardware being used to run these programs, default names for input/output

files on lines 5-10, and default options on lines 11-14. If this file is

not found by the program, default names (same as below) stored by the

program are assumed. The file as distributed contains the following lines:

EGA,GRAPH/e:5

COLOR,01,14,05,07

EPSON

HARD DISK

PLOT.DAT

JAN92.RGB,HKF81.AQU

RESTART.RES

SUMMARY.DAT

CLEAN.DAT

CLEAN.LST

METASTABLE CURVES

NO SUMMARY

DON'T PRINT ACTIVITIES

SLOPE DIFF=20

With a text editor, change these values  to match your system

configuration. Each line specifies:

1. Graphics card (EGA,CGA,HERC); GRAPH or TEXT mode for TWQ operation.

The characters after the '/' are used by the PLOT.EXE to specify the

type of graphics card your system has. The various options are listed

in the file CARD.DOC.

2. COLOR or MONOCHROME monitor (#'s give different color choices)

3. EPSON or OTHER type of printer

4. HARD or FLOPPY disc system configuration

5. Default name of plotfile generated by TWQ

6. Default names of mineral and aqueous species thermo data files

7. Default name of restart file

8. Default name of file summarizing results in tabular form

9. Default name of plotfile generated by CLEAN program

10. Default name of file that lists equilibria made by CLEAN program

11. ignored by TWQ

12. SUMMARY or NO SUMMARY - controls printing of text summary file

13. PRINT or DON'T PRINT 'SCR' file that contains computed activities for

each mineral component at each point on all computed equilibria (this file

can get lengthy!)

14. SLOPE DIFF=20 (change this value to control the lower limit that

INTERSX uses for testing for parallelism of two equilibria

-----------------------------------------------------------------

DEFINITION OF SOLUTION PHASES

All solution minerals (limited to 12 in this version) are

defined in the file SOLID.SLN. The SOLID.SLN file on the distribution disk

contains calibrations for computing activity coefficients for garnet,

plagioclase, biotite, white mica, orthopyroxene, and clinopyroxene.

Additional calibrations (up to 4 per mineral) for these and other minerals

(up to the maximum of 12) can be added as long as the entropy of mixing of

these minerals can be described by the random distribution of cations over

energetically equivalent sites (see below). The calibrations included in

this version of TWQ include:

garnet - Berman (1990); Berman & Koziol (1991)

plagioclase - Fuhrman & Lindsley (1988)

biotite - McMullin et al. (1991)

mica - Chatterjee & Froese (1975)

opx - ideal mixing (Newton, 1983)

cpx - ideal mixing (Newton, 1983)

When reported thermobarometric results, be sure to state that these models

were used. Other calibrations can be added (carefully!!) as described

below. (Note carefully that the properties of annite were derived from

Ferry and Spear's (1978) garnet-biotite data, and are thus linked to the

Fe-Mg mixing properties of garnet. If you include annite in a calculation

you MUST use the Berman(1990) garnet model for consistency. If you want to

use another calibration of Fe-Mg garnet mixing, then change the annite

properties in the file JAN92.RGB to regain consistency with Ferry and

Spear's garnet-biotite data.)

A brief description of the format of the SOLID.SLN file is given

below in order that solution parameters can be updated as each

model is refined and so that additional solution phases can be added.

Each solution phase is defined in blocks with this format:

BIOT 2         Biotite

M(3):Mg,Fe,Ti,Al-A(1):K-H(2):OH

PHLOGOPITE       Mg(3)-K(1)-OH(2)

ANNITE           Fe(3)-K(1)-OH(2)

M90      McMullin (1991)

1-13    58865.

1-14    75000.

1-23    30921.

1-24    63721.

END

IDEAL

END

The first line gives a four character abbreviation, the number of

components, and up to 20 character name for the phase. (Any abbreviation

can be used with the exception of that for plagioclase (PLAG), which has a

specific section of the program (accessed with this abbreviation) devoted

to calculation of the idiosyncracies of its solution model. The block ends

with a blank line. The second line of the block defines the different sites

and all cations assumed to occur on these sites for the purposes of the

calculation. The format is the name of the site (up to 2 characters), the

site multiplicity in parentheses, a colon, followed by a list of all

species on this site (maximum of 2 characters for each, separated by

commas); each site is separated from the next by a hyphen, except for the

last. The maximum number of sites per solution is 4, and species per site

is 5.

Next the name of each component is written on separate lines (in order for

activity coefficients to be calculated for these components, these names

must match the names of the phases as defined in the thermodynamic database

(JAN92.RGB). No spaces are allowed in these names. A maximum of 8

components is allowed for each solution phase. For each component, the name

and number of cations on each site of the phase are defined as in the

examples above.

Each solution phase may contain up to 5 alternate sets of solution

parameters which are defined on the lines after the name of the components

and before the blank line signifying the end of the definition of each

solution phase. Each set begins with a string of up to eight characters

which uniquely identifies the solution parameter set. `IDEAL' is reserved

for use with ideal solutions. Each set ends with an END line. Any lines

occurring between the first line and the END line are assumed to give

parameters (maximum of 20 ) for a general Margules solution (Berman and

Brown, 1984). Each line contains up to four entries:

a) an integer followed by a hyphen defining the number of the site this

parameter applies to, and abbreviation in terms of up to 4 subscripts of

the interaction parameter (for symmetric models: W12; for asymmetric

models: W112,W122);

b) excess enthalpy parameter (WH);

c) excess entropy parameter (WS); and

d) excess volume parameter (WV).

Also note that all parameters are in Joules per mole, not per cation, and

that if the Wohl(1953) equation is desired, the ternary constants must be

converted for use with the Berman and Brown (1984) general Margules

equation:

nRTlnGm = Sum for all parameters of QmXiXjXk..Xp/xm - (p-1)XiXjXk..Xp

where n is the site multiplicity, p is the number of subscripts

(degree of polynomial used for Gexcess) and Qm is the sum of how

many of the ijk..p subscripts are equal to the m subscript. For a

third degree polynomial (asymmetric) model, Wohl ternary

parameters (Cijk) are converted to ternary parameters given by the

above equation (Wijk) by:

Wijk = (Wiij + Wijj + Wiik + Wikk + Wjjk + Wjkk)/2 - Cijk

In other words, if you want to set the Wohl ternary Cijk = 0, then

the Wijk ternary entries in the SOLID.SLN file should be set equal to

the sum of the parameters of the three bounding binaries, all divided by 2.

-----------------------------------------------------------------

TWQ OPERATION

You are now ready to run TWQ. In the following description CR refers to the

`carriage return' key, ESC refers to the key labelled `ESC', and ^ followed

by a letter (e.g. ^Q) indicates that the `CTRL' key should be pressed at

the same time as the letter.

Type `TWQ' followed by CR. The first of several screens requesting input

will appear.

FIRST SCREEN (DIAGRAM TYPE, FILE ASSIGNMENT): Move the cursor with the

right or left arrow key or the space bar to the type of diagram you wish to

calculate.

Type ESC to continue.

The default files for input and output will appear in a window on the

screen. If you wish to change any of these files, use the up or down arrows

to move to the file to be changed, and type the new file name (including

the drive designation and directory if these are different from the current

drive and directory), followed by CR. Type ESC to continue after all files

are selected. If any of the selected files cannot be found, this screen

will repeat. Note: the only valid files for unit #5 (interactive input) are

`CON:' (the console) or `RESTART.RES' (the file containing a list of the

responses used in calculating the last phase diagram). If a file other than

these is typed, unpredictable results can be expected.

SECOND SCREEN (GLOBAL OPTIONS): Enter the elements in the desired chemical

system, separated by blanks or commas, and followed by CR. There is no

default set of elements. The phases provided in the database distributed

with this software (JAN92.RGB) are composed of the elements: Na, K, Ca, Mg,

Fe, Al, Ti, Si, O, H, and C. Any combination of these elements may be typed

in response to this question. Don't forget oxygen!

After a CR is typed, the cursor will move to the portion of the screen

where the default limits (minimum and maximum pressure in bars, temperature

in degrees Celsius, mole fraction of CO2) of the phase diagram can be

changed. If you wish to change any of these values, select the value(s) to

be changed with the cursor, and type the new value(s) (don't forget a

decimal point) followed by CR.

If you are calculating a P-T diagram and want a gas phase to be

present, move the cursor to the line showing the mole fractions of gas

species. The default values for all gas species are `0.0'. Use the cursor

or space bar to select the value(s) for any species to be changed, and type

the new value(s) (with a decimal point) followed by CR. For example, if you

want the gas to be pure H2O, change the first `0.0' to `1.0'. If you wish

the gas to be pure CO2, change the second `0.0' to `1.0'. If you want to

use a specific H2O-CO2 mixture, change both of the first two values, such

that they sum to 1. Note: If you select non-zero mole fractions for both

gas species, the program forces them to sum to 1. If you make a mistake and

need to change either or both of these non-zero values, you will need to

first reset one of the mole fractions to zero, and then type in the desired

mole fractions. This procedure must be followed in order to bypass the

built-in function that checks the sum of the mole fractions of both gas

species.

If the mole fraction of either H2O or CO2 is specified to be non-zero,

or if you are calculating a T-XCO2 or P-XCO2 diagram, the equation of state

used to calculate gas properties will be highlighted. Move the cursor to

the abbreviation of the model you wish to use for calculation of fugacities

of pure gases. If the mole fractions of both H2O and CO2 are non-zero, the

default option of non-ideal mixing using the equation of state of Kerrick

and Jacobs (1981) is highlighted. Other options can be selected by moving

the cursor to the appropriate field and typing CR. Note: The Kerrick and

Jacob mixing model does not work below about 350oC. If you choose this

mixing model, the program will set the minimum temperature of the diagram

to 350oC if the requested temperature is below this value.

Note: The thermodynamic data of Berman (1988; 1990) were derived using the

Kerrick & Jacobs equation of state for pure CO2 and H2O-CO2 mixing, and the

Haar et al. (1984) equation for pure H2O. To preserve the internal

consistency of this database these same models should be used in

calculations, although the model of Mader & Berman (1991) for pure CO2 has

been derived in such a way as to also preserve this consistency.

In the TWQ program, one cannot specify that stable segments of

equilibrium curves be plotted. To implement this option, you will need to

purchase GE0-CALC's PTAX program (see above). In addition, there is a

choice of using full mineral names or abbreviations (the default). To

change the default values for either of these options, select your choices

with the cursor and type CR.

When all choices have been made, type ESC to continue to the next

screen. Note that only the elements in the system need be specified to

calculate a phase diagram. For all other choices, the default options can

be used. At any time in the selection of the various options, the cursor

can be moved up or down to correct previous entries.

THIRD SCREEN (ENDMEMBER SELECTION): After exiting the second screen, the

program will read the thermodynamic database and put a list of endmembers

in the selected chemical system on the screen. The user must select which

endmembers are to be considered in the calculations and what their

activities are (activity = 1.0 for pure endmember). In selecting

endmembers, keep in mind that diagram complexity and time of calculation

increase dramatically as the number of endmembers considered increases.

Note: If you do not see a endmember that you wanted to consider, it means

you did not type in the proper elements on the previous screen or that the

endmember is not in the database. Type ^Q to exit the program and start

again.

Initially all endmembers are assumed to be excluded from

consideration. Move the cursor with the arrow keys to each endmember in

turn that is to be included in the calculation. Type CR to include the

endmember, or `i' to include the endmember in all reactions that are

considered. After choosing one of these options, you are prompted for the

activity of the endmember. Type the activity (don't forget the decimal

point) followed by CR, or simply type a CR to accept the default activity

of 1.0 (pure endmember). Any fixed activity that is input at this time will

be applied in addition to that based on mineral compositions supplied (see

below). If you want to remove a previously chosen endmember, move the

cursor to this endmember, and type `-'. Note: Projecting from endmembers

('p') should not be used in the TWQ program. Including endmembers ('i')

forces these endmembers in all reactions that are considered.

If you want to include all endmembers in a given calculation, type

`a'. You are then prompted to confirm this action. Type `y' or `n'.

Previous or subsequent selection of any option for a given endmember (`i'

or `-') overrides the inclusion of endmembers with this `a' option. Thus,

if you want to include all but several endmembers, you may find it more

convenient to type `a', and then remove these few endmembers with the `-'

option.

If more endmembers occur in the chemical system than will fit on one

screen (45 endmembers), use the `PgDn' key to access these endmembers and

procede with the selection as described above. Press `PgDn' to view the

next page of endmembers and `PgUp' to return to the previous page.

Once the endmember selection is complete, type ESC to continue to the

next screen.

FOURTH SCREEN (SOLID SOLUTION MODELS): This screen only appears if one of

the end-members that you included is defined as a component of a solid

solution model defined in the file SOLID.SLN (defined below).  A numbered

list of all solution phases will appear. The default state is that no

solution models are used. Type `a', 'n', or an integer to select all, none,

or a specific solution phase to be included. Default solution models (the

first option for each solution phase defined in the SOLID.SLN file) for

each solution phase are chosen if `a' is typed. If an integer is typed, the

solution model options are listed; type the number of the option desired.

When your selection is complete, type `ESC'. If any solution phase is to be

included you will be prompted for the name of the file that contains

mineral compositions (default file is ROCK.CMP). Type the name followed by

a carriage return or type QUIT if you can't remember a valid file name. The

calculation should then proceed with all equilibrium curves being displaced

with fixed activities input on the previous screen as well as activities

computed for the compositions of minerals in ROCK.CMP and the solution

parameters chosen from SOLID.SLN.

If any solution models are used, a record of these as well as the file

containing mineral compositions appears in the upper right corner of

the plot output.

A major advantage of this program is that it is set up to allow one

to see the sensitivity of results to choices of solution models. This is

accomplished most easily by editing the RESTART.RES file (contains a

record of the last TWQ run) to change the name of the solution model

desired for each solution phase to any of the models assigned in the

SOLID.SLN file. It can also be done in batch mode, by referring to

different `restart' files on each line of the BATCH.DAT file (see below).

NOTE: GE0-CALC's original PTX program had specific solution models

defined for calcite, aluminous enstatite, and hydrous cordierite

which were accessed through a solution model menu similar to that

described above. In the thermodynamic database accompanying the

present version, only those for the variable hydration of cordierite have

been retained. Two mineral names appear. If CORDIERITE, or FE_CORDIERITE

are selected the hydration model is used. If CORDIERITE (DRY), or

FE_CORDIERITE (DRY) are selected the hydration model is not used.

FIFTH SCREEN (CALCULATION OF DIAGRAM): At this point the program proceeds

to calculate the phase diagram by considering one reaction at a time. The

progress of the calculation is summarized either in TEXT or GRAPH mode,

depending on which option is specified on the first line of the PTAXCON.FIG

file.

In text mode, progress is summarized on a screen which contains three

windows. The top window indicates how many reactions are possible among the

selected phases. The maximum number that can be printed is 2 billion,

although there is no limit to the number that may be calculated. In

addition, this window displays the clock time at which the calculation

began. The middle window shows the current reaction being examined, the

clock time at the beginning of calculation of this reaction, and the status

of the reaction. The bottom window gives the running total of the number of

reactions examined, calculated, that have portions written to the plot

file, and an estimate (very approximate) of the percentage of reactions

remaining to be calculated.

In graph mode, the program's attempts to locate and follow each equilibrium

curve is viewed on the video monitor. The current equilibrium is printed at

the top of the screen, and approximate percentage remaining at the bottom

of the screen.

You may switch between modes by typing ^T or ^G. You may QUIT, PAUSE, or

STEP through the calculations by typing ^Q, ^P, or ^S. These commands take

effect after the current reaction is completed.

When the diagram is complete, the results are summarized on the final

screen which shows the clock time at the beginning and end of the

calculation, the number of reactions examined, and the number plotted. Type

ESC to finish execution of TWQ and return to DOS.

If selected on line 12 of the PTAXCON.FIG file, the results of the phase

diagram calculation may be seen in tabular form in the SUMMARY.DAT file.

Type `TYPE SUMMARY.DAT' to display the results on your monitor. The

SUMMARY.DAT file is 132 columns wide and should be printed by first typing

`MODE LPT1:132,8' to set up the printer for condensed printing. Then type

`PRINT SUMMARY.DAT' and the file will be printed on your attached printer.

See your DOS manual for further explanation of the TYPE , MODE, and PRINT

commands.

If the second to last line of the PTAXCON.FIG file contains the

string "NO SUMMARY", then the program will not write the calculated

equilibria to the SUMMARY.DAT file. For those of us who make exclusive use

of the plot output (PLOT.DAT), the size of the SUMMARY.DAT file can be

annoying, especially on batch runs.

The results may also be viewed in graphical form using the PLOT.EXE,

PRINTER.EXE, or LASER.EXE programs described below.

RERUNNING TWQ WITH MINOR CHANGES: Use of the RESTART.RES file to

recalculate phase diagrams is extremely efficient for performing repetitive

calculations with different files of mineral compositions and different

options for selected solution models. Almost any changes can be

incorporated by editing this file, the two exceptions being when a) a

different chemical system is used, and b) when a different type of diagram

is chosen (e.g. TX instead of PT). Examples of common changes to the

restart file are:

a. change P, T range of diagram

b. change phases included (0 = include; 1 = exclude)

c. change fixed activities of any phase

d. change solution models selected

e. change file containing mineral compositions

f. change fluid phase options

Rerunning TWQ with a different set of mineral compositions: The

first line after the list of minerals present in the chemical

system selected during the last TWQ run gives the name of the file

containing the compositions of all minerals in a given rock.

Change this name to that of another file with different mineral

compositions, and rerun TWQ. If no general solution models were

used on the previous run, the first 15 columns of this line will

be blank.

Rerunning TWQ to see the effect of different solution models: The

lines after that giving the name of the file containing the

mineral compositions show which solution model was used on the

previous TWQ run for each solution phase. The first character will

be "Y" or "N", depending on whether the model was used. The next

field of eight characters (separated from the previous field by 1 space)

indicates the specific solution model selected for each mineral.

These keywords are defined for each possible solution model in the

SOLID.SLN file. The last field of 20 characters (separated from the first

by 1 space) shows the name of the solution phase.

Note: the last few lines of the restart file contain the

stoichiometry of any reactions that were found to be stable on the

last run of TWQ. The default operation is that the program always computes

all possible reactions on each run. If you want to only calculate the

reactions that were stable on the last run of TWQ, change the alignment of

the last digit of the second line of the file by moving it one column to

the left.

2. Rerunning TWQ

a. Type TWQ

b. Select type of diagram <ESC>

c. Type ^R

-----------------------------------------------------------------

AUXILIARY PROGRAMS

PLOT.EXE: This program displays graphical output on video monitors using

various graphics cards. Appendix IV lists these graphic cards and the code

('/' followed by 1 or 2 characters) that should added to the PTAXCON.FIG

file or to the end of the command line. To execute this program, type `PLOT

file/?', where file is the name of the file containing the graphical output

(the default output file of TWQ is PLOT.DAT), and ? is the appropriate code

listed in Appendix IV for your graphics card. You can change the default

colors by appending four 2-digit numbers between 0 and 15, beginning with

and separated by commas, to the end of the second line of this file. The 4

numbers correspond to the default background, axis, message, and calculated

curve colors.

Once the diagram is drawn, you are prompted for the next action. To access

any of the menu commands, click with your mouse in the appropriate box, or

type the first letter of the command. The options are:

- Type ESC or 'q' to quit the program.

- Type 'b' to blowup a portion of the diagram.

Crosshairs will appear near the center of the diagram. Using a mouse or the

arrow keys, move the crosshairs to the lower left corner of the area you

want to see enlarged and type CR. Repeat this process to select the upper

right corner of the diagram. Note that the HOME, END, PGDN, and PGUP allow

diagonal movements, and holding the SHIFT key with any of the others

results in larger movements.

- Type 'o' to redraw the original diagram.

- Type 'n' to remove all labels from the diagram.

- Type 'r' to relabel the diagram.

- Type 'l' to add a label on the diagram.

- Type 'f' to superimpose another plot file on the present one.

- Type 'd' to delete a superimposed plot file.

- Type 'p' to plot symbols or draw lines.

- Type 's' to save a modified plot to a file of your choice.

PRINTER.EXE: This program makes high resolution hard copies of the

graphical output on EPSON-compatible dot matrix printers. The file

`CHARB.FON' must be in the directory.

After turning on your printer, type `PRINTER' or `PRINTER file', where file

is the name of the file containing the graphical output (the default output

file of TWQ is PLOT.DAT). The extension `.DAT' is assumed for this file and

can be omitted. If the file name is followed by a period (`.'), the program

assumes that the file has no extension. The first screen allows you to

change this file name by typing the new name followed by CR. Press ESC to

print the diagram. The complete diagram will take from 4-10 minutes to

complete depending on the speed of your computer and printer.

LASER.EXE: This program makes high resolution hard copies of the graphical

output on HP Laserjet printers. The file `CHARB.FON' must be in the

directory. It runs the same way as PRINTER.EXE described above.

CLEAN.EXE: The graphical output (PLOT.DAT) of TWQ is often difficult to

read because reaction labels overlap one another in reasonably complex

phase diagrams. The TWQ.EXE program cannot improve diagram appearance

because it deals with one reaction at a time and does not store information

regarding label positions of previously calculated reactions.

The CLEAN program can be used to improve diagram legibility. This program

reads the plot file, shifts label positions if there is space to write a

label, or replaces labels with a number if there is not enough space for

the entire label. Two output files are written: a more legible plot file

(default name is CLEAN.DAT) which can be viewed using the PLOT.EXE,

LASER.EXE, or PRINTER.EXE programs described above, and a file (CLEAN.LST)

that contains a numbered list of all reactions with the high pressure

assemblage (in P-T or P-X diagrams) or the high temperature assemblage (in

T-X diagrams) written on the left hand side of the equality sign. The

numbers of each reaction correspond to any reaction numbers that are

written in the improved plot file (CLEAN.DAT).

To run this program (after completion of the TWQ program), type `CLEAN' or

`CLEAN file', where file is the name of the plot file to be cleaned. The

extension `.DAT' is assumed for this file, and can be omitted. If the file

name is followed by a period (`.'), the program assumes that the file has

no extension.

The first screen gives you the option of changing the input/output files.

If you do not want to use the default files, move the cursor to the

appropriate field using the arrow keys or space bar, and type the new file

name followed by CR. Type ESC to continue.

Next you are prompted for the size of all labels on the cleaned-up diagram.

If you don't want the default label size (0.25), type the new label size,

including a decimal point, followed by CR. If you type a CR alone, the

default value is assumed. The lower limit for label sizes is 0.1. A few

tries running this program with different label sizes, followed by printing

the diagram with the PRINTER program will allow you to judge what label

sizes to expect.

You are next shown the various global options which can be chosen for the

new plot file. Type the number of the option you want (0 = default). If you

prefer to select options for each reaction individually, type the numbers

of each of these reactions, separated by a comma or a space. The global

options are:

0: the default option of writing all labels that will fit, replacing labels

with numbers centered on the curve, or, if numbers will not fit, replacing

labels with numbers that are removed from the curve and connected to it

with a dotted line.

-1: no reaction labels are written

-2: no reaction labels or numbers are written

-3: numbers are forced on all reactions

-4: labels are forced on all reactions

-5: labels or numbers are forced on all reactions

If any number or numbers are typed other than those above, the program

assumes that they refer to the numbers of specific reactions for which you

want to choose separate options. After a CR is typed, the program will read

all the data from the plot file and prompt you for specific options as each

curve is processed. Specification of options for individual curves is most

useful for refining the appearance of plot files on subsequent CLEAN-up

runs. Note: The program CLEAN cannot be used to process the file produced

by a previous CLEAN run; it only reads data in the specific format of

PLOT.DAT, the plot file produced by TWQ.EXE.

INTERSX.EXE: This program reads the plot file generated by TWQ

and computes an average P and T (or other axis variable) from all

intersections between curves, weighted towards those equilibria

with the highest deltaS, deltaV and smallest lnK', and which

intersect most orthogonally. To use this program after TWQ, type INTERSX,

and the output will be directed to the file PLOT.AVG as well as to the

video monitor. Some control over the results of this program are possible

by changing the slope difference on the last line of the PTAXCON.FIG

file. This value is the limit below which two equilibria are

considered to be parallel and discarded from the calculations.

Generally, use of a limit less than 20 degrees results in less

robust solutions, but in calculations with only several equilibria

a lower limit must be used to obtaint any results whatsoever.

-----------------------------------------------------------------

BATCH MODE OPERATION:

This mode of operation bypasses normal interactive mode of operation,

enabling the user to setup the computer to perform thermobarometric

calculations for a large number of samples, with different options (e.g.

end-members included, solution models, thermodynamic databases) selected.

Both the TWQ and INTERSX programs can be run in batch mode.

Edit a file call BATCH.DAT. On each line there should be 5 entries

separated by commas, with each entry < 20 characters. Although only entry#2

is required, 4 commas are expected. An example of this file is:

C:\path1\,assemb1.res,*c:\twq\jan92.rgb,*c:\twq\hkf81.aqu,suite1.dat

C:\path1\,assemb1.res,*c:\twq\jan90.hp,*c:\twq\hkf81.aqu,suite1.dat

C:\path2\,assemb2.res,*c:\twq\jan92.rgb,*c:\twq\hkf81.aqu,suite2.dat

The 5 entries indicate:

1. Path: applied in search for next 4 file names. No path is allowed

on next 4 file names unless this path is blank or unless these next

entries begin with an asterisk (*). The plot and summary output files

are written to this path. The names of the plot files and averaged

result files are taken from the first part of the names of the

composition files listed in the file which is the fifth entry

(suite1.dat in example above). The extensions are 'P' and 'A' for the

plot and averaged files, respectively, followed by the number of the

line in the batch.dat file that controls the calculation (1, 2, or 3 in

the example above).

2. Restart file name: Path from #1 ignored if first character = '*'.

3. Mineral data file: Path from #1 ignored if first character = '*'.

4. Aqueous species data: Path from #1 ignored if first character = '*'.

5. File name with list of files containing mineral compositions. Path from

#1 is applied to all of these file names, unless the first character = '*'.

To activate batch mode operation of TWQ, type TWQ/b.

The file BATCH.LOG contains info regarding status of last run. Any errors

in opening files are logged here.

To activate batch mode operation of the INTERSX program, type INTERSX/b.

-----------------------------------------------------------------

REFERENCES

Berman RG (1991) Thermobarometry using multiequilibrium calculations: a new

technique with petrologic applications. Canadian Mineralogist, v. 29, 833-855.

Berman, R.G. (1990) Mixing properties of Ca-Mg-Fe-Mn garnets. The American

Mineralogist, 75, 328-344.

Berman, R.G. (1988) Internally-consistent thermodynamic data for

stoichiometric minerals in the system Na2O-K2O-CaO-MgO-FeO-Fe2O3-Al2O3-

SiO2-TiO2-H2O-CO2. Journal of Petrology, 29, 445-522.

Berman, R.G. and Brown, T.H. (1985) The heat capacity of minerals in the system

Na2O-K2O-CaO-MgO-FeO-Fe2O3-Al2O3-SiO2-TiO2-H2O-CO2: representation, estimation,

and high temperature extrapolation. Contrib. Mineral. Petrol., 89, 168-183.

Berman, R.G., Brown, T.H. (1984) A thermodynamic model for multicomponent

melts, with application to the system CaO-Al2O3-SiO2. Geochimica

Cosmochimica Acta, 48, 661-678.

Berman, R.G., Engi, M., Greenwood, H.J., Brown, T.H. (1986) Derivation of

internally consistent thermodynamic properties by the technique of

mathematical programming, a review with application to the system MgO-SiO2-

H2O. Journal of Petrology, 27, 1331-1364.

Berman, R.G. and Koziol, A.M. (1991) Ternary excess properties of

grossular-pyrope-almandine garnets and their influence in

geothermobarometry. Amer. Mineral., 76, 1223-1231.

Chatterjee, ND & Froese EF (1975) A thermodynamic study of the pseudobinary join

muscovite-paragonite in the system KAlSi3O8-NaAlSi3O8-Al2O3-SiO2-H2O. Amer.

Mineral., 60, 985-993.

Ferry,J.M., and Spear,F.S. (1978) Experimental calibration of partitioning

of Fe and Mg between biotite and garnet. Contrib. Mineral. Petrol., 66,

113-117.

Fuhrman, M.L. and Lindsley, D.H. (1988) Ternary-feldspar modeling and

thermometry: The American Mineralogist, 73, 201-216.

Holland TJB and Powell R (1990) An enlarged and updated internally consistent

thermodynamic dataset with uncertainties and correlations: the system

K2O-Na2O-CaO-MgO-MnO-FeO-Fe2O3-Al2O3-TiO2-SiO2-C-H2-O2. J. Meta. Geol., 8,

89-124.

Kerrick & Jacobs (1981) A modified Redlich-Kwong equation for H2O, CO2, and

H2O-CO2 mixtures at elevated pressures and temperatures. American Journal

of Science, v. 281, p. 735-767.

Mader UK and Berman RG (1991) An equation of state for carbon dioxide to high

pressure and temperature. Amer. Mineral., 76, 1547-1559.

Mader UK, Berman RG (1992) Amphibole thermobarometry: a thermodynamic approach.

Current Research, Part E, Geological Survey of Canada Paper 92-1E:393-400

McMullin D, Berman RG, Greenwood HJ (1991) Calibration of the SGAM

thermobarometer for pelitic rocks using data from phase equilibrium

experiments and natural assemblages. Can. Mineral., v. 29, 889-908.

Newton, R.C. (1983) Geobarometry of high-grade metamorphic rocks. American

Journal of Science, 283-A, 1-28.

Wohl,K. (1953) Thermodynamic evaluation of binary and ternary liquid

systems. Chemical Engineering Progress, 49, 218-219.

-----------------------------------------------------------------

APPENDIX I: ENDMEMBERS IN THERMODYNAMIC DATABASE (JAN92.RGB)

ENDMEMBERS             FORMULA                   ABBREVIATION  RELIABILITY

----------             -------                   ------------  -----------

AKERMANITE             MG(1)CA(2)SI(2)O(7)              Ak         1.00

ALBITE                 NA(1)AL(1)SI(3)O(8)              Ab         1.00

HIGH_ALBITE            NA(1)AL(1)SI(3)O(8)              hAb        1.00

LOW ALBITE             NA(1)AL(1)SI(3)O(8)              lAb        1.00

ALMANDINE              SI(3)FE(3)AL(2)O(12)             Alm        1.00

ANDALUSITE             AL(2)SI(1)O(5)                   And        1.00

ANDRADITE              SI(3)CA(3)FE(2)O(12)             Andr       3.00

ANNITE                 SI(3)FE(3)AL(1)K(1)H(2)O(12)     Ann        3.00

ANORTHITE              AL(2)CA(1)SI(2)O(8)              An         1.00

ANTHOPHYLLITE          MG(7)SI(8)O(24)H(2)              Ath        2.00

ANTIGORITE             MG(48)SI(34)O(99)H(62)O(48)      Atg        1.00

BADDELYITE             ZR(1)O(2)                        Bd         3.00

BRUCITE                MG(1)O(2)H(2)                    Br         1.00

CA-AL PYROXENE         CA(1)AL(2)SI(1)O(6)              CTs        1.00

CALCITE                CA(1)C(1)O(3)                    Cc         1.00

CHRYSOTILE             MG(3)SI(2)O(9)H(4)               Chr        1.00

CLINOCHLORE            MG(5)AL(2)SI(3)O(18)H(8)         Chl        2.00

COESITE                SI(1)O(2)                        Cs         1.00

CORDIERITE (DRY)       MG(2)AL(4)SI(5)O(18)             aCd        2.00

CORDIERITE             MG(2)AL(4)SI(5)O(18)             Cd         2.00

FE_CORDIERITE          FE(2)AL(4)SI(5)O(18)             fCd        3.00

FE_CORDIERITE (DRY)    FE(2)AL(4)SI(5)O(18)             fCda       3.00

CORUNDUM               AL(2)O(3)                        Co         1.00

ALPHA CRISTOBALITE     SI(1)O(2)                        aCr        1.00

BETA CRISTOBALITE      SI(1)O(2)                        bCr        1.00

DIASPORE               AL(1)O(2)H(1)                    Dsp        1.00

DIOPSIDE               MG(1)CA(1)SI(2)O(6)              Di         1.00

DOLOMITE               MG(1)CA(1)C(2)O(6)               Do         1.00

CLINOENSTATITE         MG(1)SI(1)O(3)                   cEn        1.00

ORTHOENSTATITE         MG(2)SI(2)O(6)                   En         1.00

PROTOENSTATITE         MG(1)SI(1)O(3)                   pEn        1.00

FAYALITE               FE(2)SI(1)O(4)                   Fa         1.00

FERROSILITE            SI(2)FE(2)O(6)                   Fsl        1.00

FORSTERITE             MG(2)SI(1)O(4)                   Fo         1.00

GEHLENITE              AL(2)CA(2)SI(1)O(7)              Ge         1.00

GEIKELITE              MG(1)TI(1)O(3)                   Gk         3.00

GLAUCOPHANE            NA(2)AL(2)MG(3)SI(8)O(24)H(2)    Glc        3.00

GROSSULAR              CA(3)AL(2)SI(3)O(12)             Gr         1.00

HEDENBERGITE           CA(1)FE(1)SI(2)O(6)              Hd         3.00

HEMATITE               FE(2)O(3)                        Hm         1.00

HERCYNITE              FE(1)AL(2)O(4)                   Hc         1.00

ILMENITE               FE(1)TI(1)O(3)                   Ilm        1.00

JADEITE                NA(1)AL(1)SI(2)O(6)              Jd         1.00

KALSILITE              K(1)AL(1)SI(1)O(4)               Kls        3.00

KAOLINITE              AL(2)SI(2)O(9)H(4)               Kln        2.00

KYANITE                AL(2)SI(1)O(5)                   Ky         1.00

LAWSONITE              CA(1)AL(2)SI(2)O(10)H(4)         Lw         1.00

ALEUCITE               K(1)AL(1)SI(2)O(6)               aLc        3.00

BLEUCITE               K(1)AL(1)SI(2)O(6)               bLc        3.00

LIME                   CA(1)O(1)                        Lm         3.00

MAGNESITE              MG(1)C(1)O(3)                    Mst        2.00

MAGNETITE              FE(3)O(4)                        Mt         1.00

MARGARITE              CA(1)AL(4)SI(2)O(12)H(2)         Mrg        1.00

MEIONITE               CA(4)AL(6)SI(6)O(27)C(1)         Me         2.00

MERWINITE              CA(3)MG(1)SI(2)O(8)              Mw         1.00

MONTICELLITE           CA(1)MG(1)SI(1)O(4)              Mo         1.00

MUSCOVITE              K(1)AL(3)SI(3)O(12)H(2)          Ms         1.00

NEPHELINE              NA(1)AL(1)SI(1)O(4)              Ne         3.00

PARAGONITE             NA(1)AL(3)SI(3)O(12)H(2)         Pg         1.00

PERICLASE              MG(1)O(1)                        Pe         1.00

PHLOGOPITE             K(1)MG(3)AL(1)SI(3)O(12)H(2)     Phl        1.00

POTASSIUM FELDSPAR     K(1)AL(1)SI(3)O(8)               Kfs        1.00

SANIDINE               K(1)AL(1)SI(3)O(8)               Sa         1.00

MICROCLINE             K(1)AL(1)SI(3)O(8)               Mcr        1.00

PREHNITE               CA(2)AL(2)SI(3)O(12)H(2)         Prh        3.00

PUMPELLYITE            CA(4)AL(5)MG(1)SI(6)O(28)H(7)    Pmp        3.00

PYROPE                 MG(3)AL(2)SI(3)O(12)             Py         1.00

PYROPHYLLITE           AL(2)SI(4)O(12)H(2)              Prl        1.00

A-QUARTZ               SI(1)O(2)                        aQz        1.00

B-QUARTZ               SI(1)O(2)                        bQz        1.00

RUTILE                 TI(1)O(2)                        Rt         1.00

SILLIMANITE            AL(2)SI(1)O(5)                   Si         1.00

SPHENE                 CA(1)TI(1)SI(1)O(5)              Sph        1.00

SPINEL                 MG(1)AL(2)O(4)                   Sp         2.00

STAUROLITE             SI(7.5)FE(4)AL(18)H(4)O(48)      St         3.00

TALC                   MG(3)SI(4)O(12)H(2)              Tc         1.00

TREMOLITE              CA(2)MG(5)SI(8)O(24)H(2)         Tr         2.00

LOW TRIDYMITE          SI(1)O(2)                        lTr        1.00

HIGH TRIDYMITE         SI(1)O(2)                        hTr        1.00

WOLLASTONITE           CA(1)SI(1)O(3)                   Wo         1.00

PSEUDOWOLLASTONITE     CA(1)SI(1)O(3)                   pWo        1.00

ZOISITE                CA(2)AL(3)SI(3)O(13)H(1)         Zo         1.00

CLINOZOISITE           CA(2)AL(3)SI(3)O(13)H(1)         cZo        3.00

ZIRCON                 ZR(1)O(4)SI(1)                   Zc         3.00

CARBON DIOXIDE         C(1)O(2)                         CO2        1.00

WATER                  H(2)O(1)                         W          1.00

OXYGEN GAS             O(2)                             O2         1.00

SULFUR GAS             S(2)                             S2         1.00

HYDROGEN GAS           H(2)                             H2         1.00

APPENDIX II: TWQ PRINCIPLES AND EQUATIONS

TWQ Program Structure

The problem of calculating curves in PTX space can be broken into discrete

steps. These include:

a) specification of the PTX space and chemical system,

b) reading of the thermodynamic data base and evaluation of which phases

occur in this component space,

c) generation of reaction coefficients for all reactions in this system

using matrix methods

d) calculation of the equilibrium points on each individual curve in turn,

e) evaluation of the stability of each of these equilibrium points with

respect to all other phases in the specified system,

f) labelling of the stable assemblages on both sides of the curves,

g) output of equilibrium curves and labels to a plot file,

h) pruning the remaining list of reactions of those containing totally

metastable phases or assemblages.

Some of the more essential steps are briefly discussed below.

The location of any equilibrium point in PTX space is specified by the

condition that the Gibbs free energy change of reaction must be zero. The

program starts in the center of the diagram and, using a gradient method,

tries to locate any point with the zero free energy value. If no such

points are found, then any reactions containing the metastable assemblage

will be removed from further consideration. If an equilibrium point is

located, then the curve is followed, making use of the last two calculated

points to estimate the position of each subsequent point. Step size is

varied (as a function of the range of the axis values and of the curvature

of the equilibrium) so as to produce smooth curves. At each point on every

equilibrium curve, the stability level of the reaction assemblage is tested

against all other phases by comparing each phase's Gibbs free energy to the

chemical potential plane defined by the reaction assemblage. If the

respective stability level of any two points on a curve changes, the

location of the invariant point is found by interval halving.

All equilibria are followed until they have intersected the diagram

boundaries a minimum of two times, unless they form a closed loop within

the range of the diagram boundaries. Subsequently, the sign of the free

energy change of the reaction is checked around the edges of the diagram,

so that all segments of each equilibrium curve are located. The program

structure is such that one equilibrium is calcuated in entirety before the

next reaction is generated and followed.

Equations of State

The programs are distributed with the database derived by Berman (1988;

1990), but a number of options have been incorporated in the software that

enable users to calculate phase diagrams with alternate data sets (e.g.

Helgeson, Delany, Nesbitt, and Bird, American Journal of Science, 278-a,

1978; Robie, Hemingway, and Fisher, U.S. Geological Survey Bulletin 1452,

1978; Powell and Holland, Journal of Metamorphic Geology, 3, 327-342,

1985; Holland and Powell, 1990). Potential users must recognize that predicted

phase relations are only as accurate as the thermodynamic data used in the

calculations, and we recommend the Berman et al. database because of its high

degree of consistency with both calorimetric and phase equilibrium data. These

thermodynamic data were derived by mathematical programming analysis

(Berman et al., 1986) of all relevant data using equations of state that

were chosen for their accuracy as well as their capacity for extrapolation.

For the greatest accuracy in predicted phase diagrams, the internal

consistency of the database must be maintained, and the user must opt for

the same equations of state that were used in derivation of the database.

These are noted below, along with alternate equations that may be used in

conjunction with different databases.

The thermodynamic properties of pure H2O, CO2, and H2O-CO2 mixtures are

computed with the equations of state developed by Haar, Gallagher, and Kell

(NBS/NRC Steam Tables, Hemisphere Publishing Co., Washington, 1984), Mader

and Berman (1991), and Kerrick and Jacobs (1981), respectively. Options are

also provided to use the models of Kerrick and Jacobs (1981) for CO2 and

Holland and Powell (1990) for H2O, CO2, and H2O-CO2 mixtures.

Regardless of what database is used, the programs evaluate equilibrium

conditions by calculating the apparent Gibbs free energy of formation from

the elements (Gf) of each phase, using the equation:

Gf = Hf - TSo +   CpdT -T   (Cp/T)dT +   VdP +

Gtransition + Gdisorder + Gsolution	(1)

Hf and So are the enthalpy of formation from the elements and the third law

entropy at the reference temperature (298.15 K) and pressure (1 bar). V is

the molar volume which, for minerals, is independent of temperature and

pressure in the database of Berman et al. (1985). For a reaction, this

approximation introduces very little error for pressures up to 10 kilobars.

The expansivity and compressibility function:

VP,T / V1 bar, 298 = 1 + v1(T-298) + v2(T-298)2 + v3(P-1) + v4(P-1)2	(2)

is provided in the software so that databases which include these terms can

be used.

Heat capacity, Cp,  is calculated using the equation:

Cp = k0 + k1T-1/2 + k2T-2 + k3T-3 + k4T-1 + k5T + k6T2	(3)

where k0-k6 are fit parameters. The database distributed with this software

(Berman et al., 1985) uses only the first four terms of equation 3, thus

defaulting to the Cp equation of Berman and Brown (1985; Contributions to

Mineralogy and Petrology, v. 89, p. 168-183). The additional terms are

included so that phase diagrams can be calculated with other data sets that

use Cp equations such as proposed by Maier and Kelly (Journal of the

American Chemical Society, 54, 3243-3246, 1932) and Haas and Fisher

(American Journal of Science, 276, 525-545, 1976).

The energetic consequences of polymorphic transitions are incorporated

with an additional term in equation 1, Gtransition. In the case of lambda

or second-order transitions, this term is computed with an empirical

function (Berman and Brown, 1985) representing the variation of the heat

capacity over temperature range of the transition.

The free energy contribution resulting from temperature dependent

disordering, Gdisorder, is computed from

Gdisorder = Hdisorder - T  Sdisorder + Vdisorder(P-1)	(4)

Hdisorder and Sdisorder are represented in a manner similar to that

discussed by Helgeson and others (1978) by integrating the extended heat

capacity equation

Cpdisorder  = d0 + d1T-1/2 + d2T-2 + d3T-1 + d4T + d5T2		(5)

between a reference temperature that marks the onset of disordering and an

upper temperature limit at which the phase is fully disordered. Vdisorder

has the same temperature dependence as Cpdisorder and is calculated with

one parameter that scales the heat capacity to the volume of disorder.

Solid solution effects are added with an additional term, Gsolution, that

is calculated in a separate subroutine. Use of this subroutine is indicated

in the thermodynamic data file by assigning a four-character phase name

abbreviation which is recognized by the subroutine and specifies which

activity model to use. At present, this routine contains an activity model

hydrous cordierite (McPhail et al., 1990).

APPENDIX III: DESCRIPTION OF THE I/O FILES

The following pages describe the various files used by the program TWQ.

These descriptions are provided so that users can modify these files,

particularly the thermodynamic data file (JAN92.RGB) and output plot file

(PLOT.DAT), to suit specific needs as they may arise.

The Thermodynamic Database: Input to Unit 3

All thermodynamic and compositional data are read from the file assigned to

unit 3, which corresponds in format to the UBC/TICTM thermodynamic base.

The file is an ordinary sequential file that can be copied safely and is

easily edited with any text editor. The format is general with no

constraints on the number of the phases nor on the presence or absence of

data for a given phase.

The data base has two parts. The first identifies the elements and their

atomic weights, while the second contains thermodynamic and compositional

data for all of the solids and gases that have been included in the

complilation.

Part one

Part one identifies the atomic symbols and the atomic weights of all

elements that are used in the data base. Blank lines cannot occur within

this portion of data, but they are allowed between this portion and the

next.

The variables and format in this portion are:

NUMEL, DBASE	(Number of elements, Database abbreviation; I4,1x, A20

format)

SYMBOL(I), I = 1, NUMEL	(Atomic symbols, one for each element,

7 per line, each separated by one or

more blanks. Maximum of eight

characters per symbol.)

WEIGHT(I), I = 1, NUMEL 	(Atomic weights in grams/mole, one for

each element, 7F10 format)

The order and number of these lines are fixed; they must be exactly as

given.

Part Two

This part contains all of the thermodynamic, compositional and physical

data for every solid and gas in the compilation. The data for solids and

gases occupy separate sections and are identified by a unique heading. The

section headings must be one of the following:

****MINERAL DATA****

****GAS DATA****

****REFERENCES****

These sections are not in any definite order and can be repeated as often

as needed. The minimum number of asterisks preceeding and ending each

section heading is one; there is no maximum. The first asterisk should

occur within the first five columns of the line.

The data in each of the sections uses the same general format: lines

containing the phase name, formula, and abbreviation are followed by lines

containing the thermodynamic data.  The data is order independent, with

each line of data having an unique character designation code occurring

within the first five columns of each line.

****MINERAL DATA**** indicates that the following data refers to

thermodynamic, compositional and physical information for solid phases. The

data has the form:

PHASE NAME		FORMULA		ABBREVIATION

Standard state data (ST)

Heat capacity data (C1,C2,C3)

Transition data (T1,T2)

Disorder data (D1,D2)

Expansivity/compressibility data (V1)

Solid solution Flag (ADD)

Appendix II shows the equations of state used by the program. The segments

(standard state data, heat capacity data, transition data, disorder data,

expansivity/compressibility, solid solution flag) can be in any order. Each

segment need be included only when necessary. An exclamation point as the

first non-blank character on a line indicates a `comment line' that is

ignored by the program.

The first line for each phase must contain the phase name, mineral formula,

and abbreviation, each separated by a minimum of four spaces. The mineral

formula contains the elements (in upper case) followed by their amounts in

parantheses (see Appendix I). The numbers in parantheses may be integers or

real numbers.

The standard state segment contains ST as the first non-blank characters on

the line. The remainder of the line tabulates the Gibbs free energy of

formation from the elements (Jmol-1), the enthalphy of formation from the

elements (Jmol-1), the third law entropy (JK-1mol-1) and the molar volume

(Jbar-1), all at the reference conditions of 298.15 K and 1 bar. The

enthalpy of formation from the elements, third law entropy, and molar

volume are the minimum amount of data for a solid phase; any other data is

optional. If a phase does not have the minimum amount of information or it

has an enthalpy greater than 99999, it is ignored.

The heat capacity segment contains either C1, C2 or C3 as the first non-

blank characters. If the first characters are C1, then the remainder of the

line contains the heat capacity coefficients, k0, k1, k2, and k3 of the

equation proposed by Berman and Brown (1985). If the first characters are

C2, then the remainder of the line contains the coefficents k4, k5 and k6.

If the first characters of a line are C3, then the remainder of the line

contains the coefficents k0, k5 and k2 which corresponds to the Maier-

Kelley heat capacity equation. The use of the C3 line precludes the use of

the C1 and C2 lines.

The transition data segment provides for up to three transitions which can

be either lambda-type or first order transitions. If the first non-blank

characters are either T1, T3, or T5 (depending on the number of

transitions), the remainder of the line contains the temperature of

transition (K), a reference temperature (K) setting a lower integration

limit, the l1 and l2 lambda Cp parameters defined by Berman and Brown

(1985) in J0.5K-1 and J0.5K-2, and the assumed first order portion of the

enthalpy of transition (J). For first order transitions, only the

temperature of transition and the enthalpy of transition need be included.

If the temperature of a lambda-type transition is a known function of

pressure, the heat capacity will also be a function of pressure, and it may

be necessary to include expansivity and compressibility terms which apply

to the low-temperature polymorph. These terms are (optionally) input on a

second line identified by the characters T2, T4, or T6 (depending on the

number of transitions). The remainder of the line contains the slope of the

transition (Kbar-1), the transition volume change (Jbar-1), and the

derivatives of the volume with respect to temperature and pressure in Jbar-

1K-1 and Jbar-2, respectively.

The disorder data segment contains either D1 or D2 as the first non-blank

characters. Both D1 and D2 lines must be present. If the first characters

on the line are D1, the remainder of the line contains the d0, d1, d2, d3

and d4 terms of equation 5. If the first characters on the line are D2, the

remainder of the line contains the terms d5, d6, TMIN, and TMAX (the latter

two terms define the temperature of onset of disordering and the

temperature at which disordering is complete).

The expansivity/compressibility segment contains V1 as the first non-blank

characters on a line. The remainder of the line contains the parameters

v1(x105), v2(x105), v3(x105), and v4(x108).

The solution flag segment consists of one line with ADD as the first non-

blank characters. The remainder of the line contains a unique code name

with a maximum of 4 characters separated from ADD by at least one blank.

This code name is used as a switch to identify which part of the solid

solution subroutine is used to compute the solution properties.

****GAS DATA**** indicates that the following data is compositional and

thermodynamic properties for gases. The data for each gas has the form:

NAME		FORMULA		ABREVIATION

Standard State Data (ST)

Heat Capacity data (C1,C2)

The definition of these segments is exactly the same as the mineral data.

The volume is optional in the standard state data. The thermodynamic

properties of gases at elevated pressures are computed using special

routines discussed above.

****REFERENCE DATA**** indicates that subsequent information is text which

is ignored until a new section heading is read.

Summary Output: Output to Files Assigned to Unit 7

The output to the file assigned to unit 7 is a listing of the file

assignments, the data used, the diagram constraints, and the calculated

points of the stable equilibria.

Plotting: Output to Files Assigned to Unit 8

The information generated by the TWQ is most usefully presented as plot

output. The program writes the plot information as a series of X-Y points

on a file assigned to unit 8. This data can then be read by a other

programs (e.g. PLOT.EXE, LASER.EXE, or PRINTER.EXE) and plotted.

The format of the data written to unit 8 is as follows. The first three

lines contain information related to the axes of the diagram. The following

lines represent groups of data that are repeated for each curve, and which

define the points on the curve and its associated labels.

X-AXIS TITLE: label for the X axis of the plot

Y-AXIS TITLE: label for the Y axis of the plot

X-MIN, X-MAX, Y-MIN, Y-MAX, X AXIS LENGTH, Y AXIS LENGTH (6F10. format):

minimum and maximum axis values and scaling factors for axis lengths

NLBLS, NPTS, NSYM, NDOT: number of labels, number of X-Y points in data

set, symbol number, line-type number (4I5 format)

(X, Y, IPEN), repeated NPTS times: data points and pen control for plotting

curve (7(2F10,I2) format). IPEN values of 2 and 3 indicate moves with the

pen down and up, respectively.

(X, Y, HT, ANGLE, LABEL), repeated NLBLS times: information for plotting a

label (4F10, 5x, 120A1 format). X and Y are the coordinates of the first

letter of a label; HT is height of label; ANGLE is a negative or positive

angle measured from the horizontal in degrees. The labels consist of the

reactants and products for the reaction positioned on their respective

sides of the equilibrium. If a curve is broken by metastable segments, then

each stable segment will be labelled. X and Y are given in the appropriate

units for the diagram (bars, degrees, XCO2). HT has the same units as XSIZE

and YSIZE. The X-Y position of the label is adjusted to the height of the

labels and, therefore, is indirectly dependent on XSIZE and YSIZE.

Note: although all the details of the plot file format have been given

above so that users can add or delete data as desired, the change most

commonly made to it is simply to alter the minimum and maximum axis values

on line 3 of the data file.

Rerunning a Program: Use of Output Written to Unit 10

The file assigned to unit 10 (RESTART.DAT) contains a record of the

interactive input entered by the user on the last run of TWQ. If a diagram

is to be recalculated, this file can be assigned to unit 5 (the interactive

input unit) to avoid re-entering the input. Although this assignment can be

done explicitly, it is more convenient to simply type `^R' on the `Unit

Assignment' screen, which makes this reassignment and begins processing of

the `restart' file.

In order that users may edit this file to make changes in the next phase

diagram, details of the format of this file are provided below.

Line 1: shows the type of diagram that was last calculated.

Line 2: contains (6F10.3,I5 format) the default axis limits (XMIN, XMAX,

YMIN, YMAX), followed by the axis scale factors used for plotting (XSIZE,

YSIZE), and lastly a non-zero digit in column 65 (IRST). The non-zero value

for the IRST variable indicates that reaction coefficients of all stable

reactions on the previous run of TWQ follow at the end of this file (see

below).

Line 3: is present only for PT diagrams and shows the mole fractions of H2O

and CO2 in 2F10.3 format.

Line 4: For TX or PX diagrams, and for PT diagrams in which both H2O and

CO2 are involved, the next line indicates whether ideal (2) or nonideal (3)

gas mixing was chosen.

Line 5: shows whether only stable segments of equilibria are calculated (Y)

or whether metastable segments are followed (N).

Line 6: shows the chemical elements selected.

Line 7: is present for historic purposes only and indicates that the

following lines contain the list of all phases in the chemical system

chosen.

Each phase in the specified compositional space is listed with two

numbers. The first indicates whether the phase was included (0), ignored

(1), included in all reactions (2), or projected from (3),  while the

second shows the activity of this phase. In the restart file, the name of

the phase is listed next to these numbers in order to facilitate changes in

the selection of phases.

Next line: shows name of file containing mineral compositions

Next lines: show options for solution phases in which selected endmembers

belong

Next line: shows whether phase abbreviations were used to label curves (Y)

or whether full names were used (N).

Following this line is a list of the reaction coefficients of all stable

reactions that were calculated in the preceding run of TWQ. When re-running

the program using this file, these reactions are read from this list rather

then re-calculating all possible reactions.

If the restart file is edited and used to rerun the program, care must be

taken not to change it so that equilibria other than those listed at the

end of the file are stable. The following changes may lead to different

stable equilibria necessitating that all possible reactions be calculated

by setting the IRST variable (see lines 1 and 2 above) to zero:

-Changing the P-T, P-X or T-X space.

-Changing the composition of the fluid, pressure (for T-X diagrams), or

temperature (for P-X diagrams).

-Changing the activity of any phase.

-Choosing a different routine for calculation of gas mixing properties.

When including a phase that was suppressed on the previous diagram or

suppressing a phase that was previously included, all possible reactions

must be recalculated; change the IRST variable (see above) to zero.

-----------------------------------------------------------------

APPENDIX IV: GRAPHICS CARD OPTIONS FOR USE WITH PLOT.EXE

/A     AT&T Graphics Adaptor,                     640x400 monochrome.

/A:1   AT&T Graphics Adaptor,                     640x400 monochrome.

/A:2   AT&T Display Enhancement Board (DEB),      640x200 16-color.

/A:3   AT&T Display Enhancement Board (DEB),      640x400 16-color.

/B     Conographics ConoColor-40 Adaptor          640x400 16-color

/B:1  Conographics ConoColor-40 Adaptor          640x400 16-color

/B:2  Conographics ConoColor-40 Adaptor          512x512 16-color

/ IBM Color Graphics Adaptor (CGA)                640x200 monochrome.

/C:1  IBM Color Graphics Adaptor (CGA)           640x200 monochrome.

/C:2  IBM Color Graphics Adaptor (CGA)           320x200 4-color.

/D     Sigma Design Color-400 Adaptor,            640x400 16-color.

/E     IBM Enhanced Graphics Adaptor (EGA)        640x200 16-color.

/E:1  IBM Enhanced Graphics Adaptor (EGA)        640x350 monochrome.

/E:2  IBM Enhanced Graphics Adaptor (EGA)        320x200 16-color.

/E:3  IBM Enhanced Graphics Adaptor (EGA)        640x200 16-color.

/E:4  IBM Enhanced Graphics Adaptor (EGA, 128Kb) 640x350 16-color.

/E:5  IBM PS/2 Video Graphics Array (VGA)        640x480 16-color.

/E:6  IBM Enhanced Graphics Adaptor (EGA)        640x350 2-color.

/E:7  IBM PS/2 MultiColor Graphics Array (MCGA)  640x480 2-color.

/G     MDS Genius Display Adaptor,                738x1004 monochrome.

/H     Hercules/AST Monochrome Graphics Adaptor,  720x348 monochrome.

/L     Tseng Labs EVA/480 or NEC GB-1 Adaptor,    640x480 16-color.

/M     Microsoft mouse,                           bus version.

/M:1  Microsoft mouse,                           serial version, COMM1.

/M:2  Microsoft mouse,                           serial version, COMM2.

/M:D  Microsoft mouse driver,                    bus or serial.

/N     Number Nine Revolution Adaptor,            512x512 256-color.

/O     Toshiba 3100,                              640x400 monochrome.

/S     STB GraphicsPlus-II Adaptor,               640x352 monochrome.

/S:1  STB GraphicsPlus-II Adaptor,               640x352 monochrome.

/S:2  STB GraphicsPlus-II Adaptor,               320x200 16-color.

/S:3  STB GraphicsPlus-II Adaptor,               640x200 4-color.

/S:5  STB GraphicsPlus-III Adaptor               640x400 monochrome.

/T     Tecmar Graphics-Master Adaptor,            720x352 monochrome.

/T:1  Tecmar Graphics-Master Adaptor,            720x352 monochrome.

/T:2  Tecmar Graphics-Master Adaptor,            720x704 monochrome.

/T:3  Tecmar Graphics-Master Adaptor,            640x200 16-color.

/T:4  Tecmar Graphics-Master Adaptor,            640x400 16-color.

/U     Vega EGA+                                  640x480 16-color.

/U:1  Vega EGA+                                  640x480 16-color.

/U:2  Vega EGA+                                  720x480 16-color.

/V     Everex Edge Adaptor,                       640x200 16-color.

/V:1  Everex Edge Adaptor,                       640x200 16-color.

/V:2  Everex Edge Adaptor,                       640x400 4-color.

/W     Wyse WY-700 Graphics Display,              1280x400 monochrome.

/W:1  Wyse WY-700 Graphics Display,              1280x800 monochrome.

/W:2  Wyse WY-700 Graphics Display,              1280x400 monochrome.

/W:3  Wyse WY-700 Graphics Display,              640x400 monochrome.

/X     IBM 3270 PC,                               720x350 monochrome.

/X:1  IBM 3270 PC,                               720x350 monochrome.

/X:2  IBM 3270 PC,                               360x350 4-color.