TWQ202 (version 2.02b)

------ -------- -----

The TWQ package (version 2.02) uses the same executables as version 2.01,

together with new thermodynamic data described below.

Latest revision (TWQ202a; October, 1996): small changes to CMP.EXE

Latest revision (TWQ202b; March, 1997):

small bug fixed in INTERSX.EXE

CMP.EXE fixed to recognize CORD as cordierite

BA96a.SLN: corrected (site multiplicity of ilmenite=1, not 2)

added binary Fe-Mg spinel solution properties

Overview

--------

This file describes changes to TWQ (version 1), and is intended as an add-on

to the documentation of programs found in the file TWQ.DOC.

The main changes to this version of TWQ are in the database described below

and in a new option implemented in TWQ201 that allows only selected equilibria to

be calculated rather than all possible equilibria. To access this option, type

TWQ/S. After selecting endmembers for the calculation, the program will show

tables of all possible equilibria and ask you to select the numbers of those you

wish the program to compute. This option is useful if there are more than 3

independent equilibria and you wish to see the positions of the most robust ones

without the clutter of the others. Beware that at the present time, the program

will not perform properly on a PC if too many  equilibria are possible.

This version (2.02) of the TWQ database utilizes Berman (1988) thermodynamic

data for minerals & fluids for K-Na-CASH phases, along data for the following

minerals that have been derived by MAP analysis of relevant experimental data

discussed in detail in the references below::

1. anthophyllite, talc, rutile, as well as components of the following solid

solutions: olivine (fo, fa), orthopyroxene (en, fs, al-opx), cordierite

(Mg-, Fe-; both of which are assumed to follow the same hydration energetics),

garnet (alm, py), ilmenite (Fe-, gk),and Mg-Fe spinel.

2. hedenbergite and Mg-Fe-Al (M1) clinopyroxene mixing properties

3. provisional properties for biotite (phl, annite, siderophyllite, eastonite)

The revisions of endmember properties are based on consideration of experimental

data on solid solutions and should be used in conjunction with mixing properties

given in the files: BA96a.SLN and RECIP.SLN. Note the new formats for opx, cpx,

and biotite solutions in BA96a.sln. These formats can be generated with the

CMP program described briefly below.

Note that because some Mg endmembers have changed (fo, en, cd, py, sp), you

cannot use other Mg-, CaMg, or MgAl phases from Berman (1988) with the current

data set. Calculations are currently in progress to rectify this situation.

The references for discussion of the experimental data influencing these

revisions are:

1.  RG Berman & L Ya Aranovich (1996) Optimized

standard state and mixing properties of minerals: I. Model calibration for

olivine, orthopyroxene, cordiertie, garnet, and ilmenite in the system

FeO-MgO-CaO-Al2O3-TiO2-SiO2. Contributions to Mineralogy & Petrology (in press).

Aranovich LY, Berman RG (1996) Optimized

standard state and mixing properties of minerals: II. Comparisons, predictions,

and applications. Contrib Mineral Petrol (in press).

2. Berman RG, Aranovich LY, Pattison DRM (1995)

Reanalysis of the garnet - clinopyroxene Fe-Mg exchange thermometer.

II. Thermodynamic analysis. Contrib Mineral Petrol 119:30-42

3. Berman and Aranovich (1996a) in prep

** Note that the thermodynamic data in this version of TWEEQU are NOT identical

to the data in the above references because the version is based on simultaneous

analysis of all data, whereas the earlier works considered each set separately.

------------------------------------------------------------------------------

CMP (version 2.01)

CREATING MINERAL COMPOSITION FILES

MINERAL COMPOSITIONS

For each set of thermobarometric calculations, you must provide a file that

contains mineral compositions used by TWQ to compute displaced equilibria.

This file can have any name you like and must be edited by you so that

it contains all appropriate information about mineral compositions in one

rock or `equilibrium assemblage'. The expected compositional terms relate

to the sites that are defined in the SOLID.SLN (or BA96a.SLN) file that is described

in the TWQ.DOC file. The following is a list of the CMPTEST.CMP file that can

be used as a template for making other CMP files.

PLAG [-An-][-Ab-][-Or-]

PLAG .3537 .6300 .0163

OPX  [xMg-][xFe-][xAl2]                                 Fe3 used-calc:  .00-.08

OPX  .4899 .4492 .0608

GARN [-Gr-][-Py-][-Alm][-Sp-]  cFe3+ used-calc:  .00-.07

GARN .0288 .3183 .6353 .0175

AMPH [NaA ][K A ][v A ][MgM2][FeM2][AlM2][Fe3M][TiM2][CaM4][MgM4][FeM4][MnM4][MgM1][FeM1][MnM1][SiT1][AlT1][O O3][OHO3][F O3]

AMPH .3885 .2907 .3208 .2849 .2704 .1978 .1613 .0857 .9480 .0015 .0505 .0000 .2394 .7565 .0041 0.6079.3921 .0857 .9143 .0000

BIOT [xMg-][xFe-][F3M2][MgM1][FeM1][TiM2][AlM1][F3M1][-vM1][Al4-][-Si4][-xK-][-xOH]

BIOT .6619 .3381 .0000 .4375 .2235 .1358 .2032 .0000 .0000 .2937 .7063 0.90341.0000

CORD [xMg-][xFe-]

CORD .6743 .3232

CPX  [MgM1][FeM1][AlM1][TiM1][CaM2][MgM2][FeM2][NaM2]   - Fe3 cations = .00

CPX  .1830 .0877 .7274 .0020 .0000 .6747 .3234 .0000

Abbreviations for each mineral composition must match those used

to define the solution phase in the file SOLID.SLN (or BA96a.SLN).

The order and number of entries for each phase corresponds to the

number of components and number of sites upon which mixing takes

place in the solution phase defined in SOLID.SLN.

All compositions are input as mole fractions, which can be up to 6

digits including the decimal point. They must by aligned within

the square brackets of the appropriate component on the preceding

line.

Note: the plot file generated by TWQ will contain labels giving

the name of the file from which mineral compositions were read, as

well as the solution model used for each mineral.

For convenience in some situations, you can also put in fixed

activities for any phase component that overide fixed activities

specified during interactive input or in the restart file.

These fixed activities are input in the compositional file by

typing lines as follows (capital letters must be used):

FIXED ACTIVITY

SILLIMANITE=0.9

ORTHOENSTATITE=.5

ENDFIX

The phase names must match exactly those used in the thermodynamic data

file. If the activities are less than 0, they are assumed to be natural log

activities. Any activities input in this way are applied during the

calculation, even if activities are further reduced based on mineral

compositions that may be present in this file.

CMP.EXE provides an easy way to make a CMP file for biotite or any other of

the above solid solutions. Type in your oxides as shown in the sample file

CMPTEST.OXI - the order of oxides is not important as long as the order is

specified on the second line. The program scans the first part of each line

for key letters that identify which mineral the analysis corresponds to:

b=biotite, p=plag, o=opx, g=garnet, c=cpx, a=amph, cd=cord, i = ilmenite

THe file CMP.FIG lets you control whether there is a header line with

comments that precede the oxide list line and also the starting position of

each data line from which to search for the mineral abbreviation. The default

is having a comment line and searching from position 1. If, e.g., you don't have

a comment line and wish to search from position 7, the CMP.FIG should read

07 instead of the default

11

The program assumes the input file has the .OXI extension, and output is

directed to a file with the .CMP extension. Additional graphics file output is

sent to XXXX.p (XXXX is garn, plag, opx, cpx, amph, cord, biot depending on

which minerals are in your input file. Use TWQ's PLOT.EXE to view this file

on screen, or DXF to convert it to a DXF file that can be imported into a

number of different graphics packages.

Summary of site fraction calculations:

For gt, bi, and pyroxenes, Fe3+ can be

calculated by stoichiometry, but in general this is not recommended due the

extreme sensitivity to probe errors. Below are the mole fractions for specific

phases that need to be defined for each solution phase.

Opx: 1-site model with En-Fs-Ok (orthocorundum) endmembers.

The following mole fractions must be defined:

M(1):xEn,xFs,xOk

where, for a 6-oxygen formula,

xOk = Al/2

xFs = Fe/2

xEn = Mg/2

Nonideal Mg-Fe-Al interactions are given by Berman & Aranovich (1996)

Cpx: 2-site model with equal partitioning of Fe-Mg between sites, octahedral Al

assumed to be tshcermakitic, except for jadeite component (=Na)

The following mole fractions must be defined:

M1(1):Mg,Fe,Al,Ti-M2(1):Ca,Mg,Fe,Na

Na = Na - Fe3 (acmite)

AlM1 = Na + (Al-Na)/2  (increase AlM1 by jadeitic Al)

mgfe = Mg/(Mg+Fe2)

sum1 = 1-Ti-AlM1-Fe3

FeM1 = (1-mgfe)*(sum1)

MgM1 = (mgfe)*(sum1)

Sum2 = 1 - Na - Ca - Mn

FeM2 = (1-mgfe) * sum2

MgM2 = (mgfe) * sum2

Nonideal Mg-Fe-Al interactions are given by Berman & Aranovich (1996a)

Biotite: 3-site model assuming Al, Ti on 1 M1 site (Circone & Navrotsky, 1993).

M2(2):Mg,Fe,F3-M1(1):Mg,Fe,Ti,Al,F3,v-T(4):Al,Si

Nonideal Mg-Fe-Ti-Al mixing corrections are given by Berman & Aranovich (1996a)

Garnet

C(3):Ca,Mg,Fe,Mn

Nonideal Mg-Fe-Ca-Mn interactions are given by Berman & Aranovich (1996a)

Cordierite

M1(2):Mg,Fe

Nonideal Mg-Fe interactions are given by Berman & Aranovich (1996)

Olivine

M(2):Mg,Fe

Nonideal Mg-Fe interactions are given by Berman & Aranovich (1996)

Ilmenite

M(1):Mg,Fe

Nonideal Mg-Fe interactions are given by Berman & Aranovich (1996)

Mica

A(1):K,Na-M(2):Al-H(2):OH

Nonideal mixing corrections for K-Na are available from Chatterjee & Froese (1975).

Plagioclase

I(1):Ca,Na,K

Nonideal Ca-Na-K interactions are given by Fuhrman & Lindsley (1988).

Spinel

X(1):Mg,Fe-Y(2):Al

Nonideal Mg-Fe interactions are given by Berman & Aranovich (1996)

------------------------------------------------------------------------------

DXF.EXE (version 2.01)

To convert Ascii PLOT.DAT files to DXF files read by AUTOCAD,CORELDRW, etc.

Type DXF and answer the program queries:

Name of file: plot.dat

Plot points: 1

Scale factor: 1.0 (100.0 if a T-X diagram)

Shape: 0 (=square, 1 or more = progressively thinner rectangle)

Offset: 1 (normal position, 0 = Y position shifted down, 1 or more = Y position

shifted up progressively)

Note: the shape and offset can be used together to make connecting plots

E.g. if shape = 3 then 3 plot files will fill the space normally filled by one

if offset = 0 for the first plot file, 1 for the second, and 2 for the

third plot file, then the 3 files will plot together in the same order